Z-TRACK  Personal EME Management Software

-------------------------------------

version 3.07

November 20 , 1998

MM Software Systems

PO Box 16

Dover-Foxcroft , ME 04426 USA

(207) 965-8718

E-mail: n1bug@mint.net

WWW: http://www.mint.net/~n1bug/

------------

|  NOTICE  |

-------------------------------------------------------------------

ZT REPRESENTS A CONSIDERABLE AMOUNT OF TIME RESEARCHING AND

DEVELOPING.  IT ALSO REPRESENTS MANY HOURS RUNNING TESTS TO

VERIFY THE THEORY USED IN THE CALCULTIONS  FOR POLARIZATION

EFFECTS.  ZT IS NOT INTENDED TO BE FREE SOFTWARE.   YOU ARE

ENCOURAGED TO MAKE BACKUP COPIES FOR YOUR OWN  PERSONAL USE

BUT PLEASE DO NOT DISTRIBUTE THE PROGRAM OR ANY  PORTION OF

IT.   ONLY REGISTERED USERS WILL BE ELIGIBLE  FOR TECHNICAL

ASSISTANCE WITH ZT,  AND ONLY SUGGESTIONS  FROM  REGISTERED

USERS WILL BE SERIOUSLY CONSIDERED FOR FUTURE VERSIONS. ALL

REGISTERED  USERS  WILL BE ELIGIBLE FOR FUTURE  VERSIONS AT

REDUCED COST

-------------------------------------------------------------------

WARRANTY INFORMATION

====================

MM Software Systems warrants the physical diskette(s) in this package to

be free of defects for a period of 90 days from the date of purchase. If

a diskette  fails within  the warranty period,  return it to MM Software

Systems for a prompt replacement.

MM Software Systems specifically disclaims all other warranties,  either

expressed  or implied,  including  but not  limited to suitability for a

particular purpose.   MM Software Systems  shall not be  responsible for

loss  resulting  from  the  use  of  or  inability to  use this  product,

including incidental, consequential and other damages, with the following

single exception:  MM Software Systems will refund the purchase price of

the product (only) should it fail to perform on a particular system.

**********************************************************************

* ****************************************************************** *

* * ************************************************************** * *

* * * Please see the README.ZT file on the distribution diskette * * *

* * * for any last-minute changes or installation / use hints !! * * *

* * ************************************************************** * *

* ****************************************************************** *

**********************************************************************

------------------------------------------------------------------------

Z-TRACK 3.00                       i                 MM Software Systems

**************************** INTRODUCTION ******************************

Welcome to ZT -- a new moon/sun/star tracking  program for the VHF or

UHF EME operator. ZT places special emphasis on Spatial polarization and

Faraday rotation effects and how the two work in combination, frequently

causing 'one-way' conditions. This is in sharp contrast to other current

software which considers only the Spatial polarization and then attempts

to calculate polarization loss based on that alone. In addition to real-

time tracking, ZT also incorporates a Sked Search feature to aid in find-

ing the best schedule times. This is essentially a 'smart' common window

calculator  which  allows  you to specify such parameters as the maximum

allowable polarization loss and degradation factor, and then outputs only

times which meet your specifications.  Many other features are provided,

including an EME Calendar for very fast calculation of EME conditions for

any desired  month/day(s).   A math  coprocessor is recommended, but not

required.

This documentation  should  familiarize you with the various features

and commands available in ZT.

------------------------------------------------------------------------

Z-TRACK 3.00                      ii                 MM Software Systems

TABLE OF CONTENTS

FEATURES  OF Z-TRACK...................................................1

FILES ON YOUR DISK.....................................................3

CONFIGURING ZT   --  ZTCFG.EXE.........................................5

GETTING ON-LINE HELP IN ZT.............................................8

REALTIME TRACK SCREEN AND COMMANDS - TEXT MODE.........................9

REALTIME TRACK SCREEN AND COMMANDS - GRAPHICS MODE....................17

CURRENT BAND ACTIVITY - ZTSHWSKD.EXE..................................19

SKED SEARCH SCREEN AND COMMANDS.......................................21

MOON POSITION PRINTOUTS...............................................24

USING THE SKED EDITOR.................................................25

THE EME LINK SNR CALCULATOR...........................................27

THE EME CALENDAR FUNCTION.............................................28

THE ZT DIRECTORY......................................................29

CONVERTING LOG FILES  --  ZTCVTLOG.EXE................................30

A NOTE FOR STATIONS WITH POLARITY ROTATION............................32

ZT WARNING MESSAGES...................................................33

USER FEEDBACK.........................................................34

------------------------------------------------------------------------

Z-TRACK 3.00                      iii                MM Software Systems

*************************************************************************

************************** FEATURES OF  Z-TRACK *************************

*************************************************************************

Here is a brief summary of the major features in this release of Z-TRACK:

REALTIME MODE:

++ Real-time tracking of moon position for two locations simultaneously

with tracking of one noise source plus Cold Sky (north) for the Stn1

location.  (GRAPHICAL or text-based)

++ world map with moon  (sun optional) terminator line and moon-below-

horizon area shaded (graphics mode only).

++ Sequence timer keeps track of East/West transmitting periods.

++ Calculate Terrestrial distance and bearing to Stn2 with one keystroke.

++ Stn1 & Stn2 entry by callsign, grid square, or lat/lon.  If selected

by callsign from the directory, additional info is availble (such as

equipment and phone number).

++ Polarization Offset bar (when Stn2 tracking is on) shows spatial off-

set at a glance.  (text mode only)

++ See approximate moon elevation and polarization  conditions for most

active regions of the world with a keystroke. (text mode)

++ Calculates maximum 'non-reciprocity' in dB based on spatial offset.

++ Calculation of best transmit  polarity based  on your spatial offset

and receive polarization.

++ Standard path loss, sky temperature, DGRD indices for both 144 & 432

MHz.

++ Simple on-line logging (writes to standard ASCII text file).

++ Reminds you of upcoming skeds 5 minutes in advance (audio and visual

warning).  Shows callsign, time, and frequency of upcoming sked.

++ Fast-forward or reverse time & reset to  current  time/date with one

keystroke.  Set DOS clock from ZT if desired.

++ Optional large digits for time and AZ/EL display.

++ On-line electronic "chwat-sheet" helps you identify stations calling

when working random.

++ Instantly see who is on the band running skeds at any given time.

------------------------------------------------------------------------

Z-TRACK 3.00                       1                 MM Software Systems

SKED SEARCH MODE:

++ You supply maximum elevation for either (or both) station(s), maximum

allowable 'non-reciprocity' and Degradation, etc.  ZT gives you only

times which meet the specs.

++ Search specs can be changed while searching.

++ Searches forward or backward.

++ Reminds you of the skeds you already have to avoid 'double-skedding'.

++ compatible with sked lists exported by the SKD program.

++ Built-in sked editor for modifying or adding to the sked list,  plus

highly automated "auto-sked" feature to "drop"  skeds into your list

VERY quickly.

++ smart printer support with on-off toggle.

GENERAL:

++ EME Calendar calculates EME  conditions a  month  at a time and lets

you call up more details for specific days.

++ Lets you add entries to the DIR file & modify existing entries.

++ A utility to "freshen"  or even replace the ZT directory if you have

the latest EME.DTA file from W4ZD's 2m EME Directory.

++ Alerts & Error warnings in CW (optional).

++ Lets you shell to any of 20 user-defined external programs. Run most

other applications without having to exit ZT.

++ Multiple HELP screens.

++ EME link SNR calculator for examining equipment enhancements, etc.

++ a utility to merge ZT log files into existing PCF+2.0 databases.

++ EME Analyzer program calculates statistical data from EME logs (QSOs

vs. spatial polarity, etc) as well as statistical data for EME paths

between any two locations.

------------------------------------------------------------------------

Z-TRACK 3.00                       2                 MM Software Systems

************************************************************************

*************************** FILES ON YOUR DISK *************************

************************************************************************

The following files should be on the  ZT Distribution Diskette.   You

will  also see other  files which are part of "EME Analyzer" distrib-

ution (see AN.DOC).

ZT3.EXE          the ZT program!

ZTCFG.EXE        the ZT configuration utility

ZTCVTDIR.EXE     utility to update the ZT station directory

ZTCVTLOG.EXE     utility to convert ver 1 log files to ver 2 format

and to convert/merge log files into PCF+2.0 files.

ZTSHWSKD.EXE     utility to show a list of the skeds being run on

the band at any given time

SCHED.SRT        a chronological listing of skeds created by the

ZTSHWSKD utility.

ZTLOG.HDR        a sample PCF 'Header' file to be used with ZTCVTLOG

when doing a raw dump of log data into PCF format.

+ ZT_CFG.DAT       the ZT config file containing user-defined defaults

+ ZT3_DIR.DAT      directory file for looking up stn1/stn2 locations

+ ZT_LNK.DAT       data file used by the SNR Calculator (stores up to

5 pre-defined sets of data for quick reference).

+ UHF_DIR.DAT      432 & up directory for ZT (rename as ZT3_DIR.DAT).

+ ZT_HELP.PG1   }

+ ZT_HELP.PG2   |

+ ZT_HELP.PG3   |

+ ZT_HELP.PG4   >>> Help files for ZT's various modes and features.

+ ZT_HELP.PG5   |

+ ZT_HELP.PG6   |

+ ZT_HELP.PG7   |

+ ZT3_HELP.HLP  }

Documentation files:

NEWUSER.DOC      Quick-start instructions for new users

ZT.DOC           this file you're reading

README.ZT        Readme file for new users (Read this FIRST)

MAXNR.DOC        Text file explaining the "Max NR" index and how it

was developed

WHATSNEW.DOC     Text file outlining the changed since version 2.X

------------------------------------------------------------------------

Z-TRACK 3.00                       3                 MM Software Systems

The following files are not included on the Distribution diskette, but

are support files which can be created from within ZT.  The *.SKD file

may also be imported from the SKD program (SKD##.EXE)

*.LCP            Log conversion profiles used by ZTCVTLOG.EXE

*.SKD            (optional) a list of your upcoming skeds - used to

remind you of existing skeds so you don't accidentally

double-sked when using ZT's "Sked Search" feature

The filename will be your callsign with an extension

of .SKD

LOG.ZT           is a file used by ZT to log EME QSO's. It is a simple

ASCII file which can be printed out, edited, or

whatever.

You may also see a file called SKDBAK.ZT  after running ZT.  This is a

backup file created by the sked editor. If there are existing skeds

upon entering the editor,  they are saved to the backup file before

any changes are made.

+ NOTE: these files should not be edited or modified in any way

except through use of the ZTCFG.EXE utility.

------------------------------------------------------------------------

Z-TRACK 3.00                       4                 MM Software Systems

************************************************************************

******************** CONFIGURING ZT  --  ZTCFG.EXE *********************

************************************************************************

The first thing you will want to do is  customize  ZT 's  defaults to

your location,  monitor type,  and  Sked Search  default  specifications.

This  is  accomplished  by  running ZTCFG, and may be done in one of two

ways:

1.  ZTCFG can be run as a  stand-alone  utility.  This is the approach

to use for a first-time configuration of ZT!

2.  ZTCFG can be run from within ZT itself (F10  option 1).  This will

be discussed in more detail later, and  may be more  convenient

for making minor changes in the ZT setup as time goes by and you

upgrade your system or whatever.

When  executed,  ZTCFG will  display a list of the default parameters

currently in effect and present a menu to choose the  option(s) you wish

to change. You can change the various defaults one at a time by pressing

the letter corresponding to the item to be changed.  The process takes a

few minutes, but it will be time well spent, customizing ZT to your specs

and making it more friendly.  Here is a list of the various defaults you

can set, and a description of each:

A. -  Allows you to enter the callsign for the  home  QTH  (your call).

May be up to 12 characters long.  All characters will be forced

to upper-case by the ZTCFG utility.

B. -  Allows  you to enter your location.  You will be given a choice

between entering the grid square or latitude and longitude.  If

you select the latter, be prepared to supply the information in

Degrees, Minutes, Seconds format.

C. -  Allows you to set the maximum number of lines to be printed on

a single page when using a printer with the Sked Search mode in

ZT.  Select a size that leaves the desired bottom margin.

D. -  Allows you to enter a default frequency for the AUTOSKED mode in

Sked Search.  You can enter anything you like up to 4 characters

(well, almost...).   For example, if you are making up a list of

skeds to be  proposed via the  EME  Net and you aren't sure what

frequency  they will be  assigned,  you could use something like

"???" as a default frequency.

E. -  Allows you to enter your offset from the UTC time zone in hours.

If your computer clock is on UTC time,  enter a 0 here.   If you

are East of the UTC zone, enter the difference from UTC preceded

by a minus sign.   If you  are  West of  the UTC zone, enter the

difference from UTC with no sign.

------------------------------------------------------------------------

Z-TRACK 3.00                       5                 MM Software Systems

F. -  Allows you to  specify the  Band  (144 or 432) you want data for

most often.   ZT will  always  display  Sky Temperature data and

Degradation figures for that band when  started (the band may be

easily toggled back and  fourth between  144 and 432 while ZT is

running.   This default will only affect which band will be used

when ZT first comes up).

G. -  Allows you to enter the desired CW frequency (in Hertz) for ZT's

alert messages (ZT sends an alert message whenever it cannot find

one of its data files,  or cannot find the specified data in the

file).

H. -  Allows you to enter the desired CW speed (in WPM) for ZT's alert

messages.

I. -  Controls whether ZT will send alert messages in CW or run in the

silent mode (messages on screen only).  When On, ZT sends alerts

in CW at the specified  frequency and speed.   When Off, ZT will

not send any alert messages.

J. -  Controls whether ZT will send a warning message in CW to  remind

you of an upcoming sked.  When On, ZT's  REALTIME mode will send

the word 'SKED' 3 times five minutes  before a sked.  Whether on

or not, a flashing message will appear on the screen showing the

call of the station with whom you have a sked coming up, the time

of the sked, and the frequency.  This message will remain on the

screen for 5 minutes (until the actual start of the sked).  NOTE

that all of this only  applies if you  make a list of your skeds

available to ZT.  More on that later.

K. -  Tells ZT whether you have a color or a monochrome monitor. Select

whichever type is appropriate for your system.

L. -  Allows you to specify whether ZT's Sked Search should output data

for weekends only (UTC Saturday & Sunday) or all days of the week.

This is a startup default only,  and may be  changed while  ZT is

running.

M. -  Allows you to specify the maximum  elevation for Stn1 (home stn).

Sked Search will reject all  times  when the elevation is  above

this limit. If you can elevate fully, enter 90 here. In addition,

if the number entered here is less than 20 degrees,  Sked Search

will use 10 minute increments instead of 30 minutes. This allows

horizon-bound stations to see a more  precise  indication of the

window times. This is a startup default only, and may be changed

while ZT is running.

N. -  Allows you to specify the maximum 'Non-Reciprocity'  Sked Search

should allow.  The figure is in dB, and  Sked Search will reject

all times with higher non-reciprocity. This is a startup default

only, and may be changed while ZT is running.

------------------------------------------------------------------------

Z-TRACK 3.00                       6                 MM Software Systems

O. -  Allows you to specify the  maximum  Degradation factor (combined

affect of sky temperature  and Earth-Moon distance)  Sked Search

should allow.   The figure is in dB.   This is a startup default

only, and may be changed while ZT is running.

P. -  Allows  you to  specify  the name and path of up to twenty other

programs.  You can shell to these  programs from within ZT.  You

should specify the drive and  complete path,  and the executable

file's  extension.   (Example: if  you  have  a program  called

MYPROG.EXE in a directory called THISDIR on drive C:, you would

enter   C:\THISDIR\MYPROG.EXE    ).  The file extension MUST BE

.EXE or .COM - if you  specify any other  extension, the 'Shell'

command in ZT will  ignore the request to shell to your program.

The path may specify any drive and any directory EXCEPT the root

directory on the drive containg ZT.  You can accept the existing

path and program on a line by pressing <Return>.  If you wish to

make a change, press <BackSpace> to delete the entry, then type

your new path and filename and press <Return>.  When you are all

done specifying shell paths\programs, Press <Esc>.  NOTE: if you

need to run a batch file or DOS command from within ZT, you can

do so by specifying COMMAND.COM as one of your external programs.

You will then be able to shell directly to DOS.

When you have changed all the defaults you want to, press <Esc>.  You

will be asked whether you wish to  save the new  data or not.   Pressing

Y , y , or <Return> will save the  data and exit ZTCFG.  Pressing N or n

will exit ZTCFG without saving the changes.  Any other keystroke will be

ignored.

If ZTCFG displays only zeros and question marks for all defaults when

you run it, this means the ZT_CFG.DAT file does not exist in the current

directory or cannot be read. A new ZT_CFG.DAT file will be created after

setting all defaults (A through P), then pressing <Esc>Y (or <Esc><Ret>).

------------------------------------------------------------------------

Z-TRACK 3.00                       7                 MM Software Systems

************************************************************************

********************** GETTING ON-LINE HELP IN ZT **********************

************************************************************************

There are two inter-linked HELP systems in ZT.   A screen listing all

commands that are applicable  from a particular part of ZT can be called

up by pressing <Alt>+<H>. Note: this only works from the realtime modes,

Sked-Search mode, the utilities menu, and the sked editor.   Once in the

<Alt>+<H> Help system, you use the <PgUp> and <PgDn> keys to switch Help

pages, and the <Esc> key to leave the help system.   You can also select

any of the  commands on one of these  screens that you want to know more

about and enter the detailed Help system.  To do so, follow these steps:

Using the keyboard....

1.) use the cursor control keys (up, down, left, right arrow) to high-

light the topic you want information for.

2.) press <Return> to select it.

Using the mouse....

1.) move the mouse to highlight the topic you want information for.

2.) click the left mouse button to select it.

Read the paragraphs  that follow for  information on how to use the more

detailed Help system once you get into it in this fashion.

Pressing <Shift>+<F1>  from almost any point in the program will take

you  directly to the  detailed  on-line  Help system, which will display

information on the  part of the program you were in when you pressed the

<Shift>+<F1> keys.

There  may be more than  one screen  full of information in some Help

topics.  If so, you will see blinking  arrows at the left of your screen

to indicate  that there is  more of this topic below (or above) what you

can see at the moment.   Use the Up and Down arrow keys to scroll up and

down within a topic.

Usually, you can branch from the current Help topic to several others

related to it.   Within the Help text, any intensified text represents a

topic to which you can branch.  To do so, use the <Tab> or <Shift>+<Tab>

keys until the  topic you want is highlighted and then press <Return> to

select the new topic.   You can "back up" in sequence through previously

viewed Help topics by pressing the <BackSpace> key.

Pressing  <F1>  while in the  Help system gives you Help on using the

Help system, and <Esc> exits on-line Help.

------------------------------------------------------------------------

Z-TRACK 3.00                       8                 MM Software Systems

************************************************************************

************ REALTIME TRACK SCREEN AND COMMANDS - TEXT MODE ************

************************************************************************

When ZT is executed, it starts in  REALTIME tracking mode and you see

the main screen. ZT will come up in its simplest form, tracking the moon

for the home station only.   The ZT message will be displayed on the top

line, with the current UTC time and date immediately below it. Below this

and to the left of the screen will be a box  containing aiming  data for

Stn1.  Horizontally across the center of the screen is a box showing the

various noise and degradation factors, Moon distance, SemiDiameter, GHA,

Declination, etc.   At the bottom of the screen is a list of some of the

most common commands.  A flashing  New Moon (NM)  warning appears in the

stn1 box if the sun is within 6.5 degrees of the moon. If you have a list

of your  upcoming  skeds  available to ZT  (in a file called *.SKD), the

REALTIME screen displays a flashing reminder of an upcoming sked starting

5 minutes before sked time.  It also sends 'SKED' 3 times in CW if option

J in the setup (ZTCFG) is toggled On.  (NOTE: upcpming sked reminders are

NOT available  if the time in REALTIME track has been advanced or backed

up more than 3 minutes - see the time commands below.  If your DOS clock

is off by more than 3 minutes,  you need to set it  using  the CTRL-HOME

command described below.).

Doppler shift is calculated and displayed in the  Stn1 and Stn2 boxes

as follows:  1) if the moon is  visible at Stn1  then the  doppler shift

of that station's  echoes  is given.   2) if the moon is visible at both

stations, the net  doppler shift of stn2 as received at stn1 is shown in

the Stn2 box.   This is a  little different than some of the other moon-

tracking programs.   A quick  example:  Suppose you are W1XXX and you're

tracking the moon for your QTH (Stn1) and for G4ZZZ (Stn2).  The doppler

shift shown in the  Stn1 box is  for your own echos.  If it says 300 Hz,

then you should  find your  own  echoes  300 Hz  above your transmitting

frequency.  The doppler shift  shown in  the  Stn2  box would be the net

shift between G4ZZZ and your  location.   If it  says  140 Hz , then you

would find G4ZZZ's signal 140 Hz above his actual transmitting frequency.

ZT keeps track of the East or West transmitting sequence and displays

the appropriate ("EAST" or "WEST")  message in the upper right corner of

the screen,  adjacent  to the time/date bar.   It uses  2 or 2.5  minute

sequences,  as  appropriate for the  currently selected band by default.

You can select a 1 , 2 , or 2.5 minute  sequence and control whether the

first sequence is East or West with the F5 command discussed below.

Here are the commands available from the main screen:

<F1> - CHANGE STN1

Allows you to select another location as the primary location. You

will be prompted (in the stn1 box) to select one of 4 options:

<Esc>      select default station

enter callsign of new stn1

enter grid square of new stn1

enter lat/lon coordinates of new stn1

------------------------------------------------------------------------

Z-TRACK 3.00                       9                 MM Software Systems

Select the desired option and enter the call, grid, or coordinates

as prompted.  If you enter a callsign which cannot be found in the

ZT_DIR.DAT file,  ZT will send  'NF' in CW (if error messages are

toggled on) and ask you to press <Esc> to continue.

<F2> - CHANGE STN2

Allows  you  to  select (or change) a second station location for

tracking. A stn2 box will appear to the right of the stn1 box, and

you will be prompted to select one of 4 options:

<Esc>      cancel stn2 tracking

enter callsign for new stn2

enter grid square for new stn2

enter lat/lon coordinates for new stn2

NOTE: you may  also enter a partial callsign  preceded by

any  character that is not a letter or number (such

as  "=F6" or "?F6") to access the cheat-sheet mode.

If you do,  you will  get a list of all stations in

the directory that match the selected specification.

You can scroll up and down this list as necessary.

Select the desired option and enter the call, grid, or coordinates

as prompted.  If you enter a callsign which cannot be found in the

ZT_DIR.DAT file, ZT will  send  'NF' in CW  (if error messages are

toggled on) and ask you to press <Esc> to continue.

When stn2  tracking is  on, a  polarization offset bar appears

below the moon data box.  If the moon is visible to both stn1 and

stn2, a sliding  pointer appears  below the bar,  pointing to the

spatial  polarization offset at the present time.  Along with the

pointer is the exact offset in  degrees (marks on the bar are 2.5

degrees each, so the number  may differ a degree or so from where

the pointer is on the bar). Below this is a "Max NR" figure, which

gives an idea of the typical path non-reciprocity between stn1 and

stn2 based on the current spatial offset.  Note that one-way con-

ditions are much more likely when the  spatial  offset is near 45

degrees.  Please see the MAXNR.DOC file for a full explanation of

the  Max NR  index and  how to use it.   On a color monitor,  the

pointer, offset, and Max NR will be in green if the max NR is less

than 3 dB, in  yellow if it is  between 3 and 6 dB, and in red if

greater  than 6 dB.   The  pointer, offset,  and max NR data will

not be displayed if the moon is below the horizon for either stn1

or stn2 (or both).

NOTE:  I see that I have confused some  with the "Max NR"  figure.

This  is a  new  indicator which I developed for ZT, and I should

have provided more explanation. Starting with release 2.00, there

is a file called MAXNR.DOC  which explains this in greater detail.

It's a standard ASCII text file, just like this documentation.

If you have  polarity  rotation capability, please see "A Note

for Stations With Polarity Rotation" below.

------------------------------------------------------------------------

Z-TRACK 3.00                      10                 MM Software Systems

<F3> - STAR SELECT

Allows you to enable Sun/Csky/Star tracking.  A Star tracking box

appears to the right of the stn1/stn2 boxes, and you will be asked

to select one of 7 options:

<Esc>       cancel star tracking

<1>         track Sun & Cold Sky

<2>         track Cassiopeia & Cold Sky

<3>         track Cygnus & Cold Sky

<4>         track Saggittarius & Cold Sky

<5>         track Taurus & Cold Sky

<6>         track Virgo & Cold Sky

Select the desired object and Star tracking is enabled. The upper

set of aiming data is always for Cold Sky (north) , and the lower

set for the desired object.

<F4> - SKED SEARCH (see ALT+S below)

<F5> - SEQUENCE MENU

When ZT starts up, it selects the sequence  information based

on normal conventions for the selected band  (and will change

the sequence when you change the band).   The F5 command lets

the user override this automatic selection.

There are several options from this menu.

<A> - Same as ALT-A below,  toggles the audio/visual sequence

reminder on and off.   The only  reason I  included the

ALT-A toggle was to allow the audio/visual reminders to

be toggled quickly without having to go  through the F5

menu.

<F1> -Forces ZT to use 1 minute sequences independent of band.

<F2> -Forces 2 minute sequences.

<F3> -Forces 2.5 minute sequences.

<F4> -Puts ZT  back in  "default sequence"  mode  (sequence is

automatically selected based on the band).

<F5> -Selects  whether  the  first sequence  (beginning on the

hour) should be an EAST or WEST sequence.   This command

is disabled if F4, "Use Default sequence" is set to YES.

<ESC> - Quit and return to REALTIME tracking.

<F6> - LARGE DIGITS

Overwrites the command reminders at the bottom of the screen with

large digits for the time,  azimuth, and elevation.    Press <F6>

again to toggle this feature off and  the  screen  returns to its

previous state.  Large digits are not available when "Quick World

Data" (ALT+Q)  is on because it uses the  same area at the bottom

of the screen.  However, you can turn on "Quick World Data" with-

out having to first toggle large digits off.

<F7> - ENTER GRAPHICS MODE

See the section  of  this  documentation  titled  "REALTIME TRACK

SCREEN AND COMMANDS - GRAPHICS MODE".

<F8> - LOGGING (see ALT+L below)

------------------------------------------------------------------------

Z-TRACK 3.00                      11                 MM Software Systems

<F9> - DIRECTORY INFO

Assuming that a stn2 has been selected by  callsign (F2), all of

the directory data for that  station  will pop up in the upper -

right  corner  of the  screen.   Press <Esc> to return to active

tracking.

<F10> - UTILITIES

Takes  you to the  ZT Utilites Menu,  where you will be presented

with 8 options:

<Alt>+<H> takes  you to  page 6 of  ZT HELP,  which  describes

the 7 menu items.

<Esc>   quit and return to tracking

<1>   Configure ZT.   This  runs  ZTCFG. After you exit ZTCFG,

ZT will reset,  exactly  as if you had  exited and  then

restarted  the  program.  This ensures  that the program

is  re-initialized  to  work  properly  with   the  new

default settings.

<2>   ZT  directory  maintenance.    See "THE ZT DIRECTORY"

below.

<3>   Maintain the (optional) ZT skeds  list.  Add, modify, or

delete sked(s).

<4>   EME  link  SNR  Calculator.   See  the  section  of this

documentation covering this feature.

<5>   Create a new, updated directory  file for ZT to use with

F1/F2.  In order to run this utility you will temporarily

need a copy of the EME.DTA file from W4ZD's 2 - Meter EME

Directory, from which the new data will be taken.  If you

erase  the  old  ZT3_DIR.DAT  file  before  running this

utility,  then a  completely new  directory  file will be

constructed;  otherwise,  only data  that appears to need

updating  when  the  date  stamps  in  the two files are

compared will change.

<6>   ZT log  conversion utility.  See  the  section  on  this

utility, below.

<7>   EME Calendar utility.   Quickly calculates moon data for

upcoming months.  See "THE EME CALENDAR FUNCTION", below.

<8>   Moon printouts.  See "Moon Position Printouts", below.

<?> - CURRENT ACTIVITY

This executes the  ZTSHWSKD  utility  and provides a  list of the

skeds that are currently  in  progress  on  the  band.    See the

section on this utility below.

<Alt>+<A> - AUDIO SEQUENCE ALERTS

An optional CW  message ("EAST" or "WEST") can be  generated when

the sequence changes. ALT+A is an on/off toggle for the audio only.

When  audio  reminders  are on, a small  flashing dot (�) appears

next to the sequence at the  upper  right  corner of  the  screen.

The default at  program  startup in off.  In addition to the audio

(CW) message, the  screen  background changes and a very large "E"

or "W" is displayed  in the  center of the screen for the duration

of the CW alert.   This produces a  (sometimes annoying)  flashing

------------------------------------------------------------------------

Z-TRACK 3.00                      12                 MM Software Systems

affect that is hard to miss even if  you  can't  hear  the  audio.

Several  users  wanted a  sequence  alert that was hard to miss...

let's see  how this  works...   See the F5 command options (above)

for more sequencing options.

<Alt>+<B> - TOGGLE BETWEEN 144/432 MHZ SKY INDICES

<Alt>+<H> - ZT HELP SCREENS

Takes  you  to  page 1 of ZT HELP.  Use PgDn / PgUp to  swap help

screens, Esc to quit help. Page 1 covers REALTIME commands, page 2

SKED SEARCH  commands,  page 3 USER SHELL commands,  pages  4 & 5

SKED EDITOR, page 6 Utilities menu options, and page 7 realtime

commands for graphics tracking mode.

<Alt>+<L> - LOG STATION  (Alternate command F8)

A logging window replaces the moon data box in the  center of the

screen.  The current date and time will  automatically be entered

for you, along with the currently selected band.  In addition, if

stn2 was selected by callsign and found in the directory, the call

and grid square is also entered automatically. All that is left to

do is enter the signal reports exchanged (optional..press <Return>

to skip over) , and any comments (optioanl..press <Enter> to skip).

When the line is completed, you  will be  asked if it is OK to log

the station or not.  Press Y , y , or <Return> to log the station,

N or n if you do NOT want  the information logged.  If you choose

No, you will be asked to press <Esc> to quit ZT Logging, <Return>

to try the line again.  When a  station has been logged, you will

automatically be returned to the tracking screen. All logged data

is written to a simple ASCII file  (LOG.ZT) in the same format it

appears  in the  logging  window.  In version 2 the length of the

'Comments'  data field has been extended (22 characters vs. 14 in

version 1).  There is not  enough room in the logging box to show

all 22 characters, so the  display scrolls as you type beyond the

16th character position.  A utilty is included to convert version

1.XX log files to  the  updated format, and to merge ZT log files

into databases  used by PCF+2.0.  See the section on the utility,

below.

<Alt>+<S> - SKED SEARCH  (alternate command F4)

If stn2 is on, Sked Search starts immediately at the current time

and date, searching forward using the current  Sked Search specs.

See the Sked Search Section, and the <Ins> command below for more

information.  If  stn2 is off, the  stn2 box  will appear and you

will be prompted  to  select stn2 first.   Sked Search will start

upon  successful  selection  of stn2.  Since  Sked Search  always

starts  with the  date and time set in the REALTIME mode, you may

wish to set the date/time to the approximate time period in which

you wish to make a sked before starting Sked Search.

<Alt>+<Q> - QUICK WORLD MOON DATA

This command replaces the polarization  bar and all  other  stuff

at the bottom of the  screen with a  list of  several  geographic

------------------------------------------------------------------------

Z-TRACK 3.00                      13                 MM Software Systems

regions where the  moon is  currently  visible.  For  each region

listed, the polarization situation  relative to the  current stn1

location  is  characterized, and the  approximate  moon elevation

for that area is displayed.  An arrow  symbol  poiting up or down

shows  whether  the  moon  is still rising in that area, or if it

has passed zenith and is now  going  down.   PLEASE NOTE THAT THE

POLARIZATION  AND  ELEVATION INFORMATION SHOWN HERE IS AN AVERAGE

OF SEVERAL POINTS SCATTERED ABOUT EACH RESPECTIVE GEOGRAPHIC AREA.

It provides a  very good  general guide when operating random but

should not be used  for  scheduling a particular  station,  since

the information is an AVERAGE for a REGION and may vary  somewhat

from conditions at a specific  location.  Use  "Sked Search" mode

for  selecting  the  best  sked times.  Also note that with an XT

type  computer  (with or  without a math  coprocessor), or a  286

without a coprocessor, this  feature will  make ZT  "sluggish" in

its  response to  other commands - because to be truly useful, it

is  necessary to calculate AZ / EL and polarization loss data for

NEARLY 100 DIFFERENT LOCATIONS, thus  taking  quite a  bit of CPU

time!  By all means use this feature, but  you may wish to toggle

it on only when needed, then off  again  for general running.  If

"QUICK WORLD DATA"  is on,  pressing <Alt>-Q again toggles it off.

<Alt>+<T> - TERRESTRIAL DISTANCE and BEARING   (on/off toggle)

A window pops up in the  position of the  Sun/Star/Csky  tracking

box, showing the distance  and  azimuth  heading to the  Stn2 QTH,

as well as the  reciprocal  bearing  from  Stn2 to  your location.

If Stn2 is not on, you will be  able to  enter a  grid  square to

which  you  want  the  distance  and bearing information.  Please

note that this box will override the "Star" box if  Star tracking

is on.  In this case you will be returned to the  "Star" box when

you toggle TERRESTRIAL DISTANCE and  BEARING off.   In  any case,

you  cannot  change  the  status  of  Star  tracking  while  the

TERRESTRIAL DISTANCE and BEARING box is on. (F3 key has no effect).

<Alt>+<X> - EXIT ZT

<Alt>+<F1> through <Alt>+<F10>

<Ctrl>+<F1> through <Ctrl>+<F10> - SHELL

Shell to another program or DOS.  See the  section on  ZTCFG.EXE,

option P.  If ZT is unable to shell to  your  specified  program,

you will get an error message  and be  asked to press any key to

continue.   If you  shell  directly to DOS by way of COMMAND.COM,

you should  type 'EXIT' to return to ZT when ready.  If you shell

to  any  other .EXE or .COM file, you  should be  returned to ZT

automatically when the external process terminates. NOTE: if you

get DOS's 'Abort,Retry,Fail' massage for any reason while trying

to shell to your program - Presssing A for Abort  will TERMINATE

ZT IMMEDIATELY.  Pressing R for  Retry will do  just that: retry

the operation.  Pressing F for  Fail will  return you to ZT with

an error message and ask you to press any key to continue.

------------------------------------------------------------------------

Z-TRACK 3.00                      14                 MM Software Systems

<Ins> - SET SKED SEARCH CRITERIA

A window replaces the moon data box.  The  bottom line in the new

window shows the  current  Sked Search Parameters.   Pressing the

first letter of each item changes that item as follows:

<W>  toggles Weekend / Weekday checking

<N>  increases Max NR limit in 0.5 dB steps.

<Shift>+<N>  decreases Max NR limit

<D>  increases DGRD limit in 0.5 dB steps.

<Shift>+<D>  decreases DGRD limit

<1>  increases the  maximum  elevation limit for  stn1 in 2.5�

steps.  If stn1 max el is 20 degrees or less, Sked Search

uses 10-minute increments instead of 30.

<Shift>+<1>  decreases stn1 el limit

<2>  increases the  maximum  elevation limit for  stn2 in 2.5�

steps.  If stn2 max el is 20 degrees or less, Sked Search

uses 10-minute increments instead of 30.

<Shift>+<2>  decreases stn2 el limit

<U>  resets all parameters to default settings.  W,N,D, and 1

are user-defined defaults (set with ZTCFG). 2 is not user-

definable and defaults to 90 degrees.

<Esc>      quits Sked Search criteria  selection and  returns to the

tracking screen.

<Alt>+<P>  toggles the printer on and off.  See the section on  Sked

Search (below) for more information on use of the printer

with Sked Search.

All Sked Search criteria can also be set while in the Sked Search

mode, using the same commands.  See the section on Sked Search.

TIME COMMANDS (these are case-sensitive)

<Y> , <y>         increase  Year    , decrease  Year

<O> , <o>         increase mOnth    , decrease mOnth

<D> , <d>         increase  Day     , decrease  Day

<H> , <h>         increase  Hour    , decrease  Hour

<M> , <m>         increase  Minute  , decrease  Minute

<S> , <s>         increase  Second  , decrease  Second

<Esc>             Reset ZT to the current time set in DOS.

<Ctrl>+<Home>     Set the currently selected ZT date/time to DOS clock.

<Up Arrow>        Advance time to next half-hour interval (xx:00/xx:30)

<Down Arrow>      Back up to previous half-hour interval (xx:00/xx:30)

NOTE:  On slower computers you may notice that the time does not change

as quickly in response to keyboard commands as in  some of  the

------------------------------------------------------------------------

Z-TRACK 3.00                      15                 MM Software Systems

other real-time tracking programs.  The  reason is that some of

the programs turn off  certain  calculations while advancing or

backing up the time. This can result in confusion and/or having

to release the key(s)  for a few seconds to see all of the data

on screen updated.  Having been  annoyed by this in the past, I

decided  ZT  should  always  update ALL the information that is

shown on the  realtime  screen  whenever  it  updates ANY of it.

This  should  only be  noticable on an XT or 286 without a math

co-processor.

------------------------------------------------------------------------

Z-TRACK 3.00                      16                 MM Software Systems

************************************************************************

********* REALTIME TRACK SCREEN AND COMMANDS - GRAPHICS MODE ***********

************************************************************************

The graphics  tracking mode is  entered by pressing F7 from the text

realtime tracking mode.  (It is exited by pressing F7 again).

It will take a little while for intialization of the gragphics screen

on XT or 286 class computers.  When initialization is complete, you will

see the world map taking up approximately 2/3 of the screen.  The shaded

area on the map is where the moon is NOT visible at the  time.  A small

Roman numeral 1 (I) marks the stn1  location  on the map,  and a  Roman

numeral 2 (II) marks the stn2 location (if stn2 tracking is on).  There

is a  plus sign with an open center to mark the current position of the

moon, and a larger one to mark the position of the sun (if sun tracking

is on).  NOTE: the map will be updated once per  minute or whenever you

use any of the time commands or the F1/F2/F3 commands.

To the right of the map is a help window  listing the  commands that

can be used without  leaving the graphics  mode.   In addition to those

commands listed, any of the Date/Time  commands may be used without the

need to exit graphics mode.

Across the  bottom of the  screen is the data output window.  On the

left the azimuth, elevation, and doppler shift for stn1.   Moving right,

the next box is for az/el and doppler for  stn2 (blank when stn2 is not

activated),  and the  next is  for sun or star tracking (blank when not

activated).  For sun and star tracking, the doppler shift is replaced by

the az/el  for the cold sky  point.   The remainder  of the output data

window can take more than one form.   In its default mode, it lists the

following items :

[[[spatial offset and Max NR]]] *

GHA = Grenwich Hour Angle of the moon

DEC = Declination of the moon

R.A = Right Ascension of the moon in HOUR:MINUTES format

Rng = Earth - Moon separation in kilometers

S.D = Semidiameter of the moon in minutes of arc

DBr = dB loss cause by E-M distance (compared to perigee)

Tmp = Sky Temperature behind the moon in degrees Kelvin

DBn = dB loss from sky temp (compared to ColdSky)

DGR = total degradation from ideal EME conditions in dB

* NOTE: if stn2 is on and the moon is visible for both stn1 & stn2.

The default mode is also called "POL" mode, and can be identified by the

message "F6 : POL"  at the bottom of the help/commands list area at the

right of the screen. Pressing F6 will switch from  "POL"  mode to "HDG"

mode (in HDG mode, the polarity data will be replaced with the distance

in miles and beam heading to stn2).   Note:  HDG mode can be entered by

pressing Alt+T instead of having to step through the various F6 options

to get there.

------------------------------------------------------------------------

Z-TRACK 3.00                      17                 MM Software Systems

Another possible mode is the "CAL" mode,  which allows stations with

polarity rotation to calculate the  best transmit polarity at any given

time.   Note that CAL  mode cannot be selected with F6, but will pop up

when you hit  <Return>  for a polarity  calculation.   CAL mode  can be

cancelled either by pressing F6 or by pressing <Return> twice.

The "BIG" mode is yet  another  option.   In this mode, the time and

moon position for the stn1 location (azimuth on the left, and elevation

on the right) will be displayed in large digits.

The final mode is called "SKD"  and is activated  automatically when

you have an upcoming sked (exactly as in text mode).   SLKD mode prints

a message in the moon data area reminding you of the sked.

This can be a little confusing.   The <F6> key swithces modes in the

following sequence:

POL --> HDG --> BIG --> POL.... etc.  The shortcut to go immediately

to HDG mode is <ALT>+<T>, in order to simulate the same command in text

mode.   CAL mode can only be  accessed by pressing  <Return>,  again to

simulate the  same action in  text mode.   SKD mode switches on and off

automatically as needed.

About commands in the graphics mode...  The  following  commands act

in a similar fashion to the same commands in the text screen mode:

<F1>       - change stn1

<F2>       - change stn2

<F3>       - select/cancel sun or star tracking

<F5>       - select sequence

<F8>       - ZT logging  (alternate command ALT+L)

<F9>       - directory info (if stn2 has been selected by callsign)

<ALT>+<A>  - toggle sequence alerts

<ALT>+<B>  - Band selection

<ALT>+<T>  - Terrestrial distance and heading

ALL TIME COMMANDS

Please  read  the  section on  each of these commands in the preceeding

section of this documentation for more information.  The user-interface

to these commands is somewhat  different in  graphics mode, but  should

not be too confusing once  you have tried out each command in text mode

(a highly  recommended  practice for  new users, by the way).  The moon

data box at the lower right of the screen doubles as the dialog box for

user interface with the F1/F2/F3/F5/F8/F9 commands.

If you have a mouse...

A mouse can be used to  select  approximate  stn1 and stn2 locations

from the world map when in graphics mode.  To use this feature, position

the mouse cursor on the  location you want to select and press the left

mouse  button to make  this the active  stn2 QTH or the right button to

make it the active stn1 QTH.

You can also use the  mouse to cancel stn2 tracking or to reset stn1

to the default (home) QTH.   Simply move the mouse pointer to the RESET

button in  the extreme  upper left  corner of  the map  and  press  the

appropriate mouse button.

------------------------------------------------------------------------

Z-TRACK 3.00                      18                 MM Software Systems

************************************************************************

****************** CURRENT BAND ACTIVITY - ZTSHWSKD.EXE ****************

************************************************************************

Typing a question mark (?) from etiher of the realtime tracking modes

will  bring up this  utility,  which can show you who is on the band for

scheds at the present time and what frequency to look for them on.

You  must  keep a current  copy of the  SCHED.SKD  file (from the SKD

program in your  ZT directory for this to work).   The  ZTSHWSKD utility

checks  the  date of the  SCHED.SKD  file, and when it is newer than the

last sorted  SCHED.SRT  file created  by ZTSHWSKD it scans SCHED.SKD and

builds a new SCHED.SRT file for its use.

Whether a  SCHED.SKD  scan is  required or not, ZTSHWSKD will display

a list of skeds,  positioning you at any  skeds for the current date and

time if there are any.    You can then  scroll up and down the list from

the  initial  entry  point.   If all of the  skeds in the file are older

that the  current  date/time,  ZTSHWSKD will simply place you at the top

of the  list when it comes up.   If there  are no  skeds  at the current

time,  but there  will be  some later,  you are  positioned at the first

sked  that will  occur.  ZTSHWSKD will always attempt to put the western

station  in the first  column  (assuming it can  find both  calls in the

ZT directory).

You can run  ZTSHWSKD  as a  stand-alone  program from the DOS prompt.

Simply  typing the  program name  (ZTSHWSKD) will bring up the sked list

placing you at the  top of it.   If you want to go directly to any skeds

for a  particular  date  and time,  supply that date/time on the command

line  in the  following order :   Month, Day, Time.    For  example, the

command line ZTSHWSKD 11 9 2200  will go  immediately to the  first sked

that starts on November 9 at 2200 UTC (if there are any at that time).

ZTSHWSKD is  NOT the same as  the SKDSORT utiltiy that is distributed

by way of the BBS  (etc) despite appearances.  ZTSHWSKD has been written

specifically  to interface with ZT.

The ZTSHWSKD has other  capabilities  besides the  obvious.   For one

thing,  it will  mark any  calls that  were found in  the SCHED.SKD file

but not  in the  ZT3_DIR.DAT  file with a  hyphen (-).   It also saves a

list  of these calls  in an ASCII text file called DIRINFO.TXT.  This is

intended  to  help  you  keep  your ZT directory updated by pointing out

active stations that are not in your directory.

Another  auxiliary  function of ZTSHWSKD is the ability to mark calls

that you have not previously  worked on  EME with an  asterisk (*).   To

use this  feature  requires  that you  keep a list of  the stations  you

have worked  available to the  program in a file called MYQSOS.SKD.  For

those  of you who have not dealt with it before, the MYQSOS.SKD file was

designed to work with the SKD  program.   It is a simple ASCII text file

with one line per QSO and  3 comma-delimited fields: CALLSIGN, GRID, and

QSL RECEIVED.  For example, the line

AF9Y,EN71,R,   means you've worked AF9Y in EN71 and you have his QSL.

AF9Y,EN71,,    meany you've worked AF9Y in EN71 but don't have his QSL.

------------------------------------------------------------------------

Z-TRACK 3.00                      19                 MM Software Systems

Just  don't omit any commas - even if all  you enter is a  callsign, you

MUST put 3 commas after it!

Note: ZTSHWSKD does not require the MYQSOS.SKD file to run; it only adds

additional capabilities.   ZTSHWSKD only deals with the callsign, but if

you  also want to use the file with the SKD program you should enter all

three fields whenever possible.

------------------------------------------------------------------------

Z-TRACK 3.00                      20                 MM Software Systems

************************************************************************

********************* SKED SEARCH SCREEN AND COMMANDS ******************

************************************************************************

SKED SEARCH mode is selected  from  the  REALTIME  mode  by  pressing

ALT+S  (or F4).

The ZT Sked Search  screen  displays the date and time, Max NR, DGRD,

AZ / EL for both stations, and a New Moon Warning (if appropriate).  In

addition, ZT has the ability to remind you of skeds you have previously

made (see below).

While Sked Search is running the current month and day being  scanned

is displayed in the upper left  corner of the  screen.  If  this date is

changing but no data is  being  output, it means ZT can't find any times

that meet your selected  specifications.  It may encounter some matching

times if you let it search far enough. Or, you can 'loosen up' the search

parameters and accept a poorer time. Along the bottom of the screen is a

list of the parameters currently in effect. Each can be changed by press-

ing the first letter of the parameter  (see the <Ins> command under Main

Screen commands for details).

In addition, the  following  commands are  available when in the Sked

Search mode:

<F2>   - The F2 command  for  selecting  Stn2  works the same as it

does in the REALTIME  mode  (see above).  Upon  successful

selection of a  new QTH,  SKED SEARCH continues searching,

and  repeats a  search on  the  (approximate) time  period

that was displayed before invoking the F2  command.  Using

F2 in the  SKED SEARCH mode  will not  change the Stn2 QTH

set for the REALTIME mode (if any).

<End>  - quits Sked Search at any time.

<Esc>  - quits Sked Search at any  time.  (This has been paralleled

with the <End> key as  of  version 2.00 to avoid confusion.

My goal is to make every feature of ZT <Esc>ape-able!).

<PgDn> - searches one screen forward.

<PgUp> - searches one screen backward.

<Home> - returns to the  present  date/time  from which Sked Search

started, and repeats the search from there.

<E>,<e> - takes you directly to the Sked Editor (useful for adding a

new sked).   Any changes you  make in the  Editor  will be

reflected in the Sked Search screen upon return.

<SPACE BAR> - causes the search to halt until you hit the space bar

a second time.  This can be useful for making significant

changes to the search parameters.

<Alt>+<H> - Page 2 of ZT HELP.  (pg.1 - REALTIME, pg.2 - SKED SEARCH,

pg. 3 - USER SHELL, pgs. 4 & 5 - SKED EDITOR, pg. 6 -

UTILITIES MENU, pg. 7 - REALTIME(GRAPHICS)).

<Alt>+<P> - toggles the printer on and off. Sked Search uses a "smart"

printer control interface that keeps  track of what times

have already been printed, so  each  time will be printed

only once - even if you reverse the search direction many

times.  This function is reset whenever a new Stn2 QTH is

selected,  and  printing  will  resume from wherever Sked

------------------------------------------------------------------------

Z-TRACK 3.00                      21                 MM Software Systems

Search starts up again.  The printer  status  is shown in

the upper right corner of  the screen.  If the printer is

"not ready", the search will continue at an extremely slow

rate until the printer is made ready or toggled off.

<Alt>+<X> - Exit ZT

<Alt>+<F1> through <Alt>+<F10>

<Ctrl>+<F1> through <Ctrl>+<F10>

Shell to external application or DOS.

See the  section on these commands in the REALTIME section

(above) and in the ZTCFG.EXE section (above).

Sked Search will pause when  the  output window becomes full, awaiting

a command. Data is output in blocks of contiguous time. Whenever ZT finds

a break in the time sequence, it inserts a blank line in the output. This

helps separate times into 'windows' of opportunity.

When Sked Search has filled the  screen and is pausing  (awaiting your

choice of commands), and IF you selected the stn2 it is working with from

the directory  (by callsign) , then you  will see an "AUTOSKED is ACTIVE"

message in the upper left  corner of the  screen.   AUTOSKED gives you a

very quick  means of entering  a sked with  that station.   Use the Down

(and Up) arrow keys to  highlight the time you want to make the sked for.

You will notice that  the highlight bar skips any times that are already

occupied with skeds.  When you have the desired time highlighted, simply

press <Return> or click the left mouse button.  You will be prompted for

the desired frequency.   If you want to use the default frequency set up

by ZTCFG.EXE , just  press <Return> or click the mouse button again, and

the sked is automatically entered into your sked list. If you don't want

to use the default frequency, type the frequency you want and then press

<Return> to enter the sked.

Any parameters  set in Sked Search  remain in  effect until you change

them (either from the Sked Search screen or from the Main Screen with the

<Ins> command), or exit ZT.

As mentioned previosly, ZT can remind you of skeds already made while

Sked Search is running. This is an OPTIONAL feature. To use it, you must

keep a list of your upcoming skeds available to ZT in a file called *.SKD.

The format of the *.SKD  file is the same as *.SKD files exported by the

EME Scheduling program  (SKD##.EXE / SKD##C.EXE) using the F10,3 utility.

This makes it  convenient to  export your  skeds from one program to the

other.  If you enter your skeds in the ZT Sked Editor, then this file is

automatically created and correctly formatted.

When ZT is first started, it  looks for the *.SKD file.  If it is not

found, this feature of ZT is disabled automatically.

If you are using this feature,  ZT will place the  callsign of  those

stations you already  have  scheduled beside the appropriate time on the

Sked Search  output screen.   Skeds that do not start exactly on a 10 or

30 minute interval  (whichever  Sked Search is using) will appear on the

next earlier time.

Here are some examples:

------------------------------------------------------------------------

Z-TRACK 3.00                      22                 MM Software Systems

If Sked Search is using 10-minute intervals -

A sked starting at 0240 will be printed on the 0240 data line.

A sked starting at 0242 will be printed on the 0240 data line.

A sked starting at 0238 will be printed on the 0230 data line.

If Sked Search is using 30-minute intervals -

A sked starting at 1200 will be printed on the 1200 data line.

A sked starting at 1224 will be printed on the 1200 data line.

A sked starting at 1156 will be printed on the 1130 data line.

In addition to the callsign of the station,  the sked start time will

be displayed, and skeds that are specified as longer than 30 minutes will

have a * after the start time.

There  is  a  special  editor  included  in  ZT for adding new skeds,

deleting old ones, or modifying the existing  skeds.  See the section on

the sked editor (below) for more information.

Hopefully all of this will become clear after a while! (?)

------------------------------------------------------------------------

Z-TRACK 3.00                      23                 MM Software Systems

************************************************************************

************************ MOON POSITION PRINTOUTS ***********************

************************************************************************

You  can use  ZT to make  simple  printouts  of the moon position and

related information.   This function is accessed  through the  Utilities

menu (F10, #8).

A box appears  with the  default  starting date and time, ending date

and time, increment in minutes, and the location for which you want moon

position data.   Each item  has a highlighted  letter which you press to

change that item.  On all except the <L>ocation, pressing the highlighted

letter alone  causes the item to decrement,  while pressing <Shift> plus

the highlighted letter increments it.  If you want to change the location

the Stn1 box  will pop up  and you can  enter a  location by  any of the

usual methods : callsign, gird, or lat/lon.   You can press <Esc> at any

time to quit, or <Return> to begin printing with the current parameters.

Changing the stn1 location  while in the  printouts  screen will  not

change the stn1 location selected in other areas of the program.

The printouts created will inclule the day of the week plus the date,

azimuth and alevation of the moon,  sky temperature,  total EMR Degrada-

tion factor,  declination and GHA of the moon , a new moon warning (when

appropriate)  and notes of any  skeds that  you have  made placed on the

appropriate line in the printout.

------------------------------------------------------------------------

Z-TRACK 3.00                      24                 MM Software Systems

************************************************************************

************************ USING THE SKED EDITOR *************************

************************************************************************

The Editor is accessed either from the  REALTIME screen  through  the

Utilities (F10,3) or from the SKED SEARCH screen by pressing E (or e).

ZT has a simple editor to allow modifying the sked list without having

to exit.  It can be  accessed through the Utilities Menu (F10, option 3).

The sked editor comes  up showing a list of skeds in chronological order

and is in the Entry mode.  If there are any existing skeds when you enter

the editor, they are saved in a file called SKDBAK.ZT before any changes

are made.  If you make a serious mistake while working on the sked list,

you can exit ZT and rename the SKDBAK.ZT  file as  *.SKD to  recover the

status of the list just before the editing session.  If you have any two

skeds starting within a  25-minute  period,  the editor will display the

two  callsigns,  and the  date/times in a  box at the  lower left of the

screen.  In addition, the error code "DS"  will be  sent  in CW whenever

you invoke any  command that  initiates a  sort/display on the sked list

(<Return>,<Esc>,<F9>) - unless,  of course, you have "CW Error messages"

toggled off in the setup.

Moving around the editor  screen is  accomplished with the four arrow

keys*. When there is more than one screen full of skeds, the list scrolls

up and down as necessary. (The maximum number of skeds ZT can support is

100).

Whenever you are in the "edit mode", that is the cursor is not on the

blank entry line at the bottom,  the line  you are  currently editing is

highlighted.  While typing in data, the editor will beep if you reach the

maximum length of the field (item) you are entering.

To modify  existing sked information, use the arrow keys* to position

the cursor at the end of the item you wish to change. Then backspace over

the character(s) to be changed, and type  the new  information.  The new

information will be saved automatically  when any of the arrow keys* are

pressed (to move to a new item), or when <Return> or <Escape> is pressed.

You may update as much information as you wish. Pressing <Return> at any

time will Re-sort and re-display  the  entire  sked list, reflecting any

changes in chronological order. (this also places the cursor in column 1

of the last row and  puts the  editor back in the "enter new sked" mode).

Pressing <Escape> at  any time saves the current data and exits the sked

editor.

A new sked may be entered by placing the cursor in the first column of

the last row (a blank row), then entering the sked information, using the

left/right* arrow keys to move from one item to another.  (The cursor can

be placed in column 1, last row at any time by pressing <Return>).  When

you finish entering the new line, press <Return> again to  save the data.

To delete a sked from the list, use the up/down arrow keys to move the

cursor to the line to be deleted (cursor may be in ANY column),then press

the <Del> key.  BE CAREFUL!  ZT does not ask for confirmation on a delete

request (in the interest of simplicity and time saving). Don't press <Del>

unless you are certain you want to delete the current line.   The editor

will beep when you deleta a line.

------------------------------------------------------------------------

Z-TRACK 3.00                      25                 MM Software Systems

Remember, you can sort and re-display the entire list  at any time by

pressing <Return>, and you can save the current list and exit the editor

at any time by pressing <Esc>).

* NOTE: for those of you who are used to the editing  commands in the

scheduling program, you may use the Tab and Shift+Tab keys in

place of left/right arrow keys.  The function is identical.

ZT Help can be accessed from the editor by pressing Alt+H.  Pages 4

and 5 of ZT Help cover use of the sked editor.

<F9> - while in the editor,  pressing <F9>  will  bring  up AZ/EL and

polarization data for each sked. This information replaces the

Notes column.  The original notes ate NOT modified in any way,

and may be  redisplayed  at any  time by  pressing <F9> again.

The exact information displayed is AZ/EL for stn1 (home), AZ/EL

for stn2, spatial polarization offset, and Max NR.  ALL DATA IS

FOR THE SCHEDULE START TIME.  While the AZ/EL/pol data is being

displayed it is not possible to tab into the  Notes  column to

make changes.  This is a safety  feature to  prevent inportant

notes from accidentally being modified or lost when you cannot

see them!  When ZT if first started, and at  most other  times,

the <F9> toggle should be instantaneous.  The exception is when

you have modified one or more schedules since the last time you

viewed the AZ/EL/pol data using <F9>. In this case, the program

will need to recalculate all of the data for this command, and

a short delay may be noticed on the slower machines.  When the

AZ/EL/pol display is on, there will  be a short delay whenever

a new sked is added or an existing  sked is modified, in order

to update the  data.  If this is  objectionable, simply toggle

<F9> off  before  editing the sked list.  A single dash (-) in

the Notes  column when viewing AZ/EL/pol data indicates ZT was

not able to find this station in the directory so no data could

be calculated.

<Alt>+<P> - PRINT SCHEDULES

Pressing Alt+P while in the editor will print the entire list

of skeds on your printer.  Please be sure the printer is on line

and ready to go.  Either the notes or the sked moon data will be

displayed, depending on the current setting of  <F9>.   If sked

moon data is on, one additional  column  will be  added to  the

printed schedule list that does not appear on the  screen - the

DGRD (total degradation from ideal EME conditions from sky temp

and Earth-moon distance).

------------------------------------------------------------------------

Z-TRACK 3.00                      26                 MM Software Systems

************************************************************************

********************** THE EME LINK SNR CALCULATOR *********************

************************************************************************

The EME link SNR Calculator is accessed  through the  utilities  menu

(F10,4).

Data can be entered for two  stations,  and the Signal-to-noise ratio

for each station's  echoes as  well as the SNR for station A as heard by

station B  and vice  versa is  calculated.  Of course,  the  S/N  ratios

calculated would be for "normal" conditions with no  Faraday,  libration,

ground-gain,  etc.   Nevertheless, the SNR  calculator  is a good way to

find out what  areas  of  your  station  could be improved  and what the

benefits of an upgrade would be.

You can change  any of  the  items in  the  upper portion of both the

STATION A   and   STATION B   boxes.  Use  the arrow  keys to move about

in this area, and when the cursor is positioned on  the item you want to

modify, simply type the  new  data  and  press  <Return>.   Only numeric

data is valid, and  therefore  nothing  else will be accepted.  Striking

any key other that the digits 0-9 and  decimal points will do absolutely

nil.  All of the  SNR  ratios, etc.  will  be recalculated automatically

when you press <Return>.

There are 5 pre-stored data  sets in  the disk file ZT_LNK.DAT.  You

can replace the stored data with your  own  data by entering the desired

data and pressing <F1>. The new data will overwrite the current data for

the active data  set (listed just below the main header on this screen).

You can go to the next (or previous) stored data set with the <PgUp> and

<PgDn> keys.   <Esc> exits back to the utilities menu.

------------------------------------------------------------------------

Z-TRACK 3.00                      27                 MM Software Systems

************************************************************************

********************** THE EME CALENDAR FUNCTION ***********************

************************************************************************

This function is accessed through the Utilities Menu(F10,7).

The calendar screen will come up with a  menu bar  across the top, on

which the next 12 months are listed in MM/YY format, and the last option

is "Other".   You may select one of these options by using the right and

left cursor  movement (arrow) keys, then pressing <Return> on the option

you choose.   If you  choose  "Other", you will be prompted to enter the

month  and  the  year - otherwise,  calculations will begin immmediately

when you hit <Return>.

After a moment, the data for each day will be written into the appro-

priate squares on the calendar (any  days  which  would  require a sixth

partial row  will wrap  around  onto the top line of the calendar).  The

declination  and  EME degradation factor will be shown for each day, and

indicators for  New Moon (NM),  Full Moon (FM), Apogee, and Perigee will

be placed on the appropriate days.

If you wish to see detailed data for one particular day, press <PgDn>

then use the cursor movement  keys to position the cursor on the desired

day and press <Return>.   After a  second or two,  the  data will pop up

showing detailed  declination,  sky noise,  Earth-Moon  separation, days

away from  Perigee/Apogee  and  New Moon/Full Moon,  and any  local moon

events (rises, sets) for the currently selected stn1  location (normally

this will be your location).   Press  any key  when you are done viewing

the moon data for this day.

You  can  print  a  month  of data by pressing <P>, or a year (the 12

months  listed)  by  pressing <Y>.   Press <Esc> to  exit  back  to  the

Utilities Menu.

If you have a mouse...

A mouse can be used to control many actions in the EME Calendar mode.

You can  select the month to be calculated when you enter the Calendar.

You can then use the right mouse button to enter into "detail mode" and

then  position the mouse  cursor on the day you want data for and press

the left mouse button to  calculate (then use the right button again to

escape).   If a mouse is detected by ZT, instructions for using it will

be on the top of the screen.

------------------------------------------------------------------------

Z-TRACK 3.00                      28                 MM Software Systems

************************************************************************

************************** THE ZT DIRECTORY ****************************

************************************************************************

The  ZT station directory  is  used by  several parts of the program,

including  the F1 & F2 commands,   and the Sked Editor.   The following

information  pertains to the ZT directory,  which  resides  in the file

named ZT3_DIR.DAT.

Keeping the directory  up-to-date  is very important.   ZT gives you

two ways of doing this:

1).  The ZTCVTDIR  utility  lets you update your ZT directory from

a copy of  W4ZD's  EME  Directory.   To run ZTCVTDIR you must

place a  copy of the  EME.DTA file  from the EME directory in

the directory where you have  ZTCVTDIR.  When ZTCVTDIR is run,

it will do one of  two things - if you  do not have  a  valid

ZT3_DIR.DAT file,  one will be  created  which  will  contain

data  on all of the stations in the EME directory - otherwise,

the  ZT3_DIR.DAT file  will be updated  only where  necessary,

adding any new stations or changing data for others.

2).  The built-in F10 option 2 utility in ZT allows you to manually

add stations, delete old ones, and change data for any station.

Simply type the callsign of  the  station you want to work on,

and ZT  will check  to see if  the station  is already  in the

directory.  If not it will ask you to fill in the data for the

new station.  Just enter the  appropriate data in each box and

press <Return>.  Then press the up or down arrow to move on to

the next item.  Note that if you enter a grid the latitude and

longitude will automaticlly be entered for you, and vice versa.

When you have  everything entered,  press <Esc> and answer the

"Write data to file ?" prompt with Y to save the data.

If the station selected  is already in the directory,  you may

edit the data as required and then  save it by pressing  <Esc>

and answering Y.   You can  also delete  the station  from the

directory by  pressing  <Del> and then Y.   Please not that it

is  NOT  necessary to  delete a station  and then  enter a new

record in  the event  of a callsign change  -  simply edit the

callsign box and save the record.

When you are  finished working on the directory and have saved

the last modified record,  press  <Esc>  instead of entering a

callsing to exit.

ZT also does automatic  background  checking to ensure that both the

grid square  and  lat / lon  coordinates are  in each  record of the  ZT

directory.   This occurs  every time  directory information is requested

by any part of the program and will automatically calculate and save any

missing information.   (Although to be honest,  I don't know  how such a

situation would arise,  what with  the built-in  calculation  of grid or

coordinates when entering a new station.   It would have to come from an

incomplete entry in the EME.DTA file when running ZTCVTDIR).

------------------------------------------------------------------------

Z-TRACK 3.00                      29                 MM Software Systems

************************************************************************

***************** CONVERTING LOG FILES -- ZTCVTLOG.EXE *****************

************************************************************************

This utilty can be accessed through the Utilities Menu(F10,6)  or may

be run as a stand-alone program.

ZTCVTLOG.EXE does three things:

1)  converts old log files from ZT ver 1.XX to the slightly different

format used in version 2.0.

2)  converts  version 2.0 log files to "raw" PCF+2.0 format.  Many of

ZT's users are running PCF+2.0 with the 2m EME Directory and other

databases.

3)  merges ZT version 2 log files into existing PCF+2.0 EME QSO data-

bases.

You will find it easy to use ZTCVTLOG once you get used to it, but it

requires a bit of explanation.

CONVERTING ZT ver 1.XX LOG FILES TO ver 2.0 FORMAT

This is the simplest option of all.  When the ZTCVTLOG  screen  comes

up, select option 1, then specify the name of the old version 1 log file

to convert and the name of the new  version 2 log file to be  created at

the prompts.  The old  and new  filenames can be the same if desired, in

which case the old one will be renamed to ZTLOG.OLD.

CONVERTING ZT ver 2.0 LOG FILES TO "RAW" PCF FORMAT

Select option 2 from the main screen, then  option 1 from the  second

list.  Then specify the filenames as prompted. The filename ZTLOG.DTA is

suggested for the new PCF file, although any file name with an extension

of .DTA will work (with a little extra  effort  on your part).  Once the

file has been converted, you must build an index for the new PCF database.

If you used ZTLOG.DTA as the filename,  place this new file and the file

called  ZTLOG.HDR  from your ZT distribution disk in the directory where

you want the PCF database to be, then run  PCFIX on the new file.  If you

used a different filename for the .DTA file, then you will have to rename

ZTLOG.HDR to match your filename  (*.HDR). This option simply copies the

data directly from the ZT log file  into a  database with the same field

definitions, etc.

MERGING ZT ver 2.0 LOG FILES INTO EXISTING PCF QSO DATABASES

You must supply a copy of your database's .HDR file and its .DTA file

to use this option.

This is by far the most complex of the three  options.   First select

option 2, then option 2 again from  the  second  list.  Then specify the

name of your PCF database to merge the QSOs into. The first time you run

this option you will have to specify which  fields  in your database you

want the log data copied into.  You  will  be shown a list of the fields

that exist in your database, as  well  as those used in the ZT log files.

You will first be asked to supply a format for the date to be inserted in

your database.  Select the desired one from the list by its corresponding

number. Next you will be asked what field to place each of the items from

the ZT log into. Respond to each question by specifying the number of the

------------------------------------------------------------------------

Z-TRACK 3.00                      30                 MM Software Systems

field in your PCF  database where the indicated data should go.  (if you

don't have a place for any of the data to go in the PCF file, just answer

with a 0 and this  item  will not be transferred.  After ansering all of

these questions you will be asked whether you want to save the answers to

these questions in a  configuration file (*.LCP) for future use.  If you

will be using this option from time to time,  you should  answer Yes  so

that you will not have to  go through  the whole process again each time.

Once a Log Conversion Profile  (*.LCP) file has  been saved, you will be

asked whether you want to use it each time you attempt to merge QSOs into

this  particular  database.  You  only  need  to  go  through  the  whole

interrogation  process  again  if  you  Clone  or  modify  your  database

definition in some way.

You will be asked  for the name of the ZT log file to merge into your

PCF  database.  (the usual  filename is LOG.ZT, but you may have renamed

some of your log files from ZT). Once the process is completed, copy the

new *.DTA file into the  directory  where PCF normally looks for it, and

run PCFIX on it to create a new index.  You will probably want to run a

sort on the data as well.

NOTE: no field-length  checking is  performed.  If you  try to cram a

field with 10 significant characters (significant characters are anything

other than spaces) into a field in your database that only  holds 8, the

data will be truncated to fit.  This should not be a  problem except for

possibly the date field.  Be sure to pick an option that matches what is

in your database or at least something that is not too long to fit in the

available space.

------------------------------------------------------------------------

Z-TRACK 3.00                      31                 MM Software Systems

************************************************************************

*************** A NOTE FOR STATIONS WITH POLARITY ROTATION *************

************************************************************************

ZT has a special feature for stations using polarity (Z-axis) rotation.

It is possible to calculate the best transmit polarity based on the best

receive polarity and the spatial polarization offset at any given moment.

When stn2 tracking is on (and if the moon is  visible to both  stations),

the message '<Return> = RX / TX  polarity  calculation' is displayed near

the bottom of the screen.  Simply  hit <Return> to activate this feature,

and enter the receive polarity  when prompted.  The receive polarity must

be between -90 (vertical rotated CCW) and +90 (vertical rotated CW), with

zero representing horizontal  polarization.  Don't forget the '-' sign if

the receive polarity is CCW.  ZT will then calculate and display the best

transmit polarity at the moment, and the net Faraday rotation.

The calculated  results  may or  may not be valid for very long.  Any

significant change in spatial offset  (even if the best receive polarity

remains unchanged) means it is time to do the calculation again.  By the

same token, any change in optimum  receive polarity (whether the spatial

offset is changing or not) is reason to re-calculate Faraday and transmit

polarity.  To recalculate,  just  hit <Return> and enter the best receive

polarity again.

The line displaying the  receive  polarity, Faraday, and best transmit

polarity can be cleared by striking <Return> twice.

NOTE : in graphics mode, the net Faraday will not be displayed, but the

optimum receive and transmit polarity settings will be there.

------------------------------------------------------------------------

Z-TRACK 3.00                      32                 MM Software Systems

************************************************************************

************************** ZT WARNING MESSAGES *************************

************************************************************************

The following is a list of possible ZT error  conditions and  warning

messages (in CW).  Warning messages will only be generated if toggled On

using ZTCFG.

long beep - the ZT_CFG.DAT file was missing or could not be read at

program startup!  It is possible to run ZTCFG to create

a new CFG file (see CONFIGURING ZT above).

-- or --

a long beep  followed  by an  on-screen message and the

DOS prompt means that ZT encountered a  critical  error

while running, and therefore has  terminated.  Normally

you  should  NEVER  see this....  but if you do, PLEASE

JOT  DOWN THE ERROR CODE INFORMATION SHOWN NEAR THE DOS

PROMPT AND SEND IT TO ME SO I CAN LOOK FOR AND (I HOPE)

EXTERMINATE THE BUG!   The  more  information  you  can

provide as to where you were in the program at the time,

the higher my chances of finding the culprit.

*  '?'       - the indicated file could not be found!

'DS'      - the Sked Editor has detected 2 skeds starting within

a 25 minute period!

'SOF'     - Sked OverFlow. You have more than 100 skeds. If you get

this error,  you  will  NOT be  permitted  to  edit the

partial sked list.

'NF'      - the indicated data is not in the DIR file!

'NR'      - the printer is not ready.

'MEM'     - not enough memory to Shell to your application. You may

also get this  message  when  trying to run  ZTCFG from

within ZT.

'ERR'     - non-specific  error  trying to  Shell  (or run ZT.CFG).

'NA'      - the selected feature is not available (this is usually

caused by a missing *.DAT file).

* exception: if the *.SKD file is not  found at  startup,  NO  message

is generated since this is an OPTIONAL feature.

------------------------------------------------------------------------

Z-TRACK 3.00                      33                 MM Software Systems

************************************************************************

**************************** USER FEEDBACK *****************************

************************************************************************

I hope you like version 3.00 of Z-TRACK.   I've  tried to  provide  a

program that takes a more logical approach to the polarity situation, and

aids in selecting good sked times.  There are additional things which may

be added in future versions, of course.    Got an idea for version 4 ???

Got a problem with this version ???  Please take a moment to let me know

about it... I will  do my  best to  accomodate as  many  suggestions  as

possible in a  future  version.   I CAN ONLY ACT ON YOUR IDEAS IF I KNOW

ABOUT YOUR IDEAS !!!  Please call or write for information on our growing

line of software products, both commercial and public domain.

MM Software Systems

PO Box 16

Dover-Foxcroft, ME  04426 USA

(207) 965 - 8718

E-mail: n1bug@mint.net

WWW: http://www.mint.net/~n1bug/

------------------------------------------------------------------------

Z-TRACK 3.00                      34                 MM Software Systems