A BRIEF TUTORIAL ON Z-TRACK's  "MAX NR" INDICATOR

---------------------------------------------------

Paul Kelley

April, 1993

I've been asked about the "Max NR" figure generated in Z-TRACK by

several users of the program.  Most of the questions were just plain

old  "Huh ???" , indicating that I really goofed.  Max NR, as far as

I know,  had never been used in any  moon-tracking software prior to

the first release of Z-TRACK.   I developed it as a new indicator to

supplement the  "Spatial Polarization Offset"  so often seen in such

software.   I firmly believe it is a much more meaningful and easily

interpreted indicator of polarization conditions.   So, here goes my

best shot at removing the shroud of mystery from the Max NR index !!

I've had a go (or two, or...)  at explaining this  verbally, without

the kind of success I was hoping for.  MAYBE I can do better writing

it down (???).

--------------------------------------------------------------------

Z-TRACK MaxNR Index              1                       Paul Kelley

THE PRE-BASICS

----------------

I suspect few people will argue these days that the polarization

of an  incoming  signal  with  respect  to the  polarization of the

receiving  antenna  makes a tremendous  difference in  how well the

signal can be heard.   A signal  perfectly  aligned in polarization

with the  receiving  antenna system will suffer no loss of strength

due to polarization mis-alignment.  A signal arriving perpendicular

to the  polarization plane of the antenna  (90 degrees to it)  will

suffer  the greatest loss - How much  depends on many  factors, but

for a  typical modern yagi it is  generally taken as between 20 and

30 dB.   In between these extremes  the loss varies logarithmically

with  the  cosine of the  angle  (by angle I mean the difference in

degrees  between the  polarization plane of the incoming signal and

that of the antenna).  A formula for this is:

Loss (dB) =  20 log(cos  = the angle in degrees.

This formula assumes a  perfect yagi  with no  unwanted response to

signals of the opposite polarization.  It is VERY close up to about

the 18 or 20 dB point,  but  beyond  that needs a correction factor

to keep the result down to a realistic level.

That's all well and good  for two  antennas of like polarization

pointing toward each other along the Earth's surface.   But what of

two antennas of like  polarization  pointing  off into space?   The

situation  becomes a little more complicated.   In my own crude way

let me try to help you  visualize  what I mean.   Suppose you had a

large round ball  sitting in  front of you.   If you took two samll

model yagis and  planted one on the  very top of the ball  (be sure

to align the  elements so they are  parallel to the  surface of the

ball  directly below the antenna)  so that it is  pointing straight

away  from you  toward a  distant  wall,  and the other on the left

side of the ball (elements  parallel to the  surface  of the  ball)

pointing to the same distant wall, You will have the  basis of this

visualization.    Both  antennas are  horizontally  polarized  with

respect to the surface of the ball (simulated Earth),  right?   But

stand back and sight along the boom of each yagi, toward that wall.

If you can  visualize a signal  leaving each antenna and travelling

to the the wall (simulated moon),you will see that the polarization

planes of the two signals  hitting the wall are actually 90 degrees

out  of  sync with  each  other.   This  difference is what we call

"Spatial" Polarization Offset!  Gee Whiz!

The geometry involved on an  EME path is a little more  complex,

what with elevation angles and so on.   The point of reference used

for such  calculations is  generally the  Earth's polar axis, but a

complete  discussion of the mechanics involved is beyond the intent

of this discourse (and is a fantastic way to get a headache).  Here

is the basic formula used to calculate a spatial polarizatoion  for

an antenna with respect to the Earth's polar axis:

--------------------------------------------------------------------

Z-TRACK MaxNR Index              2                       Paul Kelley

(sin L * cos E - cos L * cos A * sin E)

P = ATN  ---------------------------------------

(cos L * sin A)

where L = Latitude of station           {footnote 1}

A = Azimuth of antenna

E = Elavation of antenna

P = Polarization angle

The spatial polarization offset (read 'difference') between any two

stations is simply P1 - P2, but keeping it within a range of -90 to

+90 takes additional steps.

MOVING ON TO THE REAL WORLD

-----------------------------

For some time we have been accustomed to seeing a  "Polarization"

or  "Spatial Polarization Offset"  figure in  moon-tracking programs

that  track the moon  for two stations.   This figure is in degrees,

with  some programs  keeping it within a range of -90 to +90 degrees

while others allow it to approach 180.  Along with this, many of the

programs have also tried to include a  "Polarization Loss" figure in

dB, with 0 dB indicating a polarization offset of 0 (or 180) degrees

and 25 dB or more indicating a polarization offset of 90 derees.

It doesn't  take a rocket scientist to  figure out that this just

cannot be the case.  We've all made some fine EME QSOs when software

told us to  expect 20+ dB loss of signal based on polarization!   So

what's the deal here?   The problem is that programs like that don't

consider the affects of Faraday  rotation.   The fact is that due to

Faraday rotation in the ionosphere signals can arrive at the receive

antenna at ANY polarization, reguardless of the spatial polarization

relationship  between the two stations.    Given that consideration,

the next thing  we're  confronted  with is the  argument that ALL of

this spatial polarization business should be  disreguarded since the

unpredictable  Faraday is going to modify  the  polarization anyway.

In other words we "make our skeds blindly and take our chances".

BUT - and this is a B-I-G but - the ionosphere has another unique

little  property that should make us re-think the situation.  Simply

put, a signal  polarization which rotates in a given direction  when

passing  through  the  ionosphere on its way to the moon will rotate

the same amount in the same direction (from the observer's viewpoint)

on the return  trip.   You can  visualize it this way:   Suppose you

were standing  behind the reflector on your EME antenna and sighting

down the  boom to the moon.   Further  suppose for a moment that you

could see the  polarization plane of the signal leaving your antenna

and  traveling to the  moon and back  again to the antenna.   If you

sent  out a  burst of RF and  observed  that the  polarization plane

shifted  45  degrees to the right  (clockwise) on the way out to the

moon, your first  assumption  would probably be that the signal will

rotate in the  opposite direction (from your perspective) on its way

back, thus  "un-twisting" itself and arriving perfectly in line with

the elements on your antenna.   But the ionosphere doesn't see it in

--------------------------------------------------------------------

Z-TRACK MaxNR Index              3                       Paul Kelley

that way.   In fact the  polarization  plane will rotate by the same

amount in the same direction  (clockwise) from your vantage point on

the  return trip - thus making it now  90 degrees  out of  alignment

with your antenna.

So what?   Consider this:  If you add a second  station whose QTH

is far enough  away  from  yours so  that the  spatial  polarization

offset between  the two  of you is 45 degrees, something interesting

begins to take shape.   Suppose you  transmit and the Faraday causes

the polarization plane of your signal to rotate 45 degrees clockwise

by  the  time  it  reaches  the  other  station.   It started out at

0 degrees (the spatial polarization of your antenna) and was twisted

45 degrees clockwise, so it arrives at his antenna perfectly aligned

with it (his spatial  polarization is 45  degrees greater than yours,

don't forget).   Sounds  like the  Faraday is  doing us a  big favor,

right?  WRONG!   Now suppose the  other  station  transmits a string

of O's back to you  (having heard your signal very well by virtue of

the perfect polarization alignment).   The polarization plane of the

signal starts out at 45 degrees  (his spatial polarization) and then

gets  rotated 45 degrees  clockwise  on its way to you,  making  the

polarization of the signal  90 degrees in  "spatial" terms.   Yikes!

Your  antenna is  still at 0 degrees "spatial" polarization, and his

signal  is coming in at  90 degrees!   Unless you're a  BIG GUN, you

just won't be  able to  hear him.   Good grief, one-way propagation!

WHEREAS THE ABOVE SITUATION IS AVOIDABLE...

---------------------------------------------

Fortunately,  there  is a way  to  figure out what times are most

likely to yield  frustrating one-way  conditions in advance.   Going

back to the example above...   If the spatial offset between the two

stations were 0 deegrees,  then  Station A  transmitting to  station

B would  result in a clockwise twisting of  45 degrees,  arriving 45

degrees  out of alignment with his antenna.  On the return trip, the

exact  same  relationship  holds.   If  the  Spatial  offset were 90

degrees, then  station A  transmitting to  station B would result in

a signal at 45 degrees  "spatial polarization"  being received by an

antenna with a  polarization of 90 degrees,  for a  difference of 45

degrees.   On the return trip,  90 degrees starting from his antenna

gets twisted  clockwise to  135 degrees  (135 degrees is  equivalent

to -45 degrees.   It  should  always be  kept in a  range of  +/- 90

degrees by adding or  subtracting  180  degrees  as  necessary.  180

degrees is  physically the same as 0 degrees), which is an offset of

45 degrees with reguard to your antenna.   In  either of these cases

the signal will be  mis-aligned by  the same  amount (45 degrees) at

both stations.  There will be a resulting loss of 3 dB, but at least

it's reciprocal (and 3 dB isn't as bad as 20+ dB).

I could  (and probably should)  sit here  and go  through various

possible combiantions of "spatial offset" and  Faraday  rotation for

a L-O-N-G time.  I'm not going to, but I would  like to bring up one

other point before going on.  If one works away at this problem long

enough  (I've been  fretting  away at it for 5 years now),  one then

begins  to think  in terms of "windows of opportunity".  Considering

--------------------------------------------------------------------

Z-TRACK MaxNR Index              4                       Paul Kelley

the original  45 degree spatial  offset again...  The  absolute best

that can be achieved (under normal circumstances)  in this case is a

Faraday of 0 degrees or 90 degrees.   This would  indeed result in a

reciprocal path with 3 dB loss  of signal at both ends.  BUT, if the

Faraday changes by only  15 degrees in either  direction, the signal

misalingment  at one  station will become  30 degrees (1.24 dB loss)

and at  the other station  60 degrees  (6 dB).   Notice  how quickly

the signal  falls off at one end of the path for very small  changes

in prevailing  Faraday - thus a  small  "window of opportunity"  for

reciprocal  conditions,  what with  Faraday being such a  changeable

entity.   On the other  hand, consider either of the perfect cases -

that is  either  spatial 0, Faraday 0    or  spatial 90, Faraday 90.

Under  these  circumstances the path is reciprocal with 0 dB loss at

either end.   If the Faraday  changes 15 degrees in either direction

the reciprocity will remain, and the signal will drop by only 0.3 dB

at  both  stations.    Indeed,  the  Faraday  must  shift 45 degrees

one  way or the other  before the  signal  drops to  -3 dB  and even

then  it is still reciprocal.   Hence a greater "window of opportun-

ity".

The bottom line is  that one-way  conditions are much more likely

to occur when the  spatial  offset is in the  vicinity of 45 degrees

and drops to  nearly 0 when  the  offset is 0 or 90  degrees.   Data

collected over the past few years  clearly shows that  statistically

the chances of  completing  an  EME QSO drop  off sharply  when  the

spatial offset  approaches  45 degrees,  especially for the  smaller

stations who just don't have the signal to spare. Many have  clearly

benefitted in  the past few  years by careful consideration of these

polarization issues.

A BETTER WAY ???

------------------

Considering  the spatial  offset in degrees is all  right up to a

point, but it can become a nuiscance.  I couldn't help thinking that

what we really  wanted to  know was how  non-reciprocal a given path

could get at a particular time.   The spatial offset information was

nice, but I would find myself wanting to know more.  When working up

sked times,  I often wondered  about the actual non-reciprocity that

might come into play. Hmm, the offset is 38.2 degrees, the cosine of

38.2 is ahhh...  and the log of that is ummm.....    Blast and drat!

After a few false starts (not to mention sleepless nights) I came up

with the "Max NR" figure.   This  gives a good  approximation of the

non-reciprocity that is  likely to be  encountered  based on a given

spatial offset.   The name  is somewhat  of a misnomer, since Max NR

does not always  give the  absolute  maximum non-reciprocity that is

possible for a given time.   It gives  a precise  maximum  only  for

offsets of 0, 45,  or 90 degrees.   At other offsets it gives a very

good  approximation  of  typical  non-reciprocity  values  that  are

actually observed on EME.  Spatial offsets near 45 degrees will show

the highest Max NR figures, as offsets in that range are the most

likely to cause severe one-way conditions.

--------------------------------------------------------------------

Z-TRACK MaxNR Index              5                       Paul Kelley

One must bear in  mind that  this is  neither a  worst-case nor a

best-case indication.  However, it is  a very good  indicator of the

typical non-reciprocities seen on EME and correlates quite well with

EME logs  analyzed  to  date.   What the  Max NR  does is  provide a

convenient and user-friendly way of weeding out times that have been

statistically  proven  to have the lowest chance for success.

A VERY GOOD "FIT"

------------------

When I devised the new Max NR index, I needed  to see how well it

would  fit with  observed EME  results,  compared to  other  methods.

I analyzed several EME logs for percentage of QSOs vs spatial offset

and  Max NR.   The  graphs  of  QSOs vs. spatial  offset  showed the

anticipated  dip around 45 degrees (+/- a fair bit), but it was less

than dramatic, all  things  considered.   I next plotted the QSOs vs.

Max NR and  found an  amazing  correlation.   I was not prepared for

the results.   All of the  logs checked showed 90 percent or more of

the QSOs  occurred when the  Max NR  was  less than 3 dB!   It was a

smooth  curve with a very sharp  drop for low but  increasing values

of Max NR,  leveling out as it  approached 25 dB.  There was usually

a small "anomaly" at  25 dB,  caused by the method used to round off

(or limit)  Max NR  indices that  soar above  that point.   Remember

my  previous  comment  about  the  equation  for  dB  loss needing a

correction  factor above  18 or 20 dB?   I am very pleased  with the

results obtained.

Here is an attempt at reproducing the graphs in text form  (for a

typical log):    [These  are  lacking in  resolution - it's tough to

reproduce graphics on an 80x25 text screen!!!].

EME QSOs VS SPATIAL OFFSET

#   100-|*

90-|**  *

O    80-|*****

F    70-|******* **

60-|*************   **                                 * *    *  **

E    50-|********************                       *   ****************

M    40-|**********************                 ************************

E    30-|*********************** **           **************************

20-|****************************   **  ****************************

Q    10-|***************************************************************

S     0-|***************************************************************

O    -------------------------------------------------------------------

s       0     10     20     30     40     50     60     70     80     90

SPATIAL OFFSET (DEGREES)

--------------------------------------------------------------------

Z-TRACK MaxNR Index              6                       Paul Kelley

EME QSOs VS MAX NR

#   1000-|*

900-|*

O    800-|**

F    700-|**

600-|***

E    500-|****

M    400-|*****                                       round-off anomaly

E    300-|********

200-|**********

Q    100-|**************                                              **

S      0-|**************************************************************

O    -------------------------------------------------------------------

s        0    2    4    6    8   10   12   14   16   18   20   22   24

Max NR (dB)

RECOMMENDATIONS FOR USE

-------------------------

Having  introduced  the  concept  of  "Max NR" , here is a brief

comment on what I have used it for.  When making skeds I start with

the smallest stations I want to run with,  scheduling them when the

Max NR is at its lowest (typically less than 1 dB).  Then I move up

to progressively larger  stations,  accepting somewhat poorer times

as the size of the station increases.   If you make  many skeds you

will find that it is not  possible to schedule  them all at "ideal"

times!   Generally speaking,  I try to keep the Max NR on all skeds

under 3 dB.  I rarely suffer the "one-way blues" any more, although

it does  happen on  rare  occasions.   (For every rule, there is an

exception).

Spatial Offset (and hence Max NR) varies drastically from day to

day as the  declination of the  moon  changes.   Hint: if you can't

find a time with low Max NR on a given day, try a few days later or

earlier.   Sometimes, you have to give up...  During the recent EME

expedition to KC6, I ran a statistical analysis of the path from my

QTH to KC6 and found that the  lowest  Max NR for the entire common

moon window during that month was 12.6 dB!  Arghhh!!!

EXCEPTIONS - QSOs NEAR 25 dB MAX NR

-------------------------------------

Any discussion on the  subject of  one-way EME conditions  would

not be complete (or fair) without some comments about why some good

QSOs are indeed possible at 45 degrees spatial offset (or 25 dB Max

NR).   You will have noticed that while the graphs above show a dip

in the  number of  EME QSOs in this  region,  it certainly does not

drop to zero.  There are at least 3 reasons for this.

Perhaps the most obvious exception is when the two stations have

enough power and combined antenna gain to overcome very substantial

losses.  It's certainly possible to give up many dB and  still work

some of the super-stations.

--------------------------------------------------------------------

Z-TRACK MaxNR Index              7                       Paul Kelley

As mentioned before, a situation exists when the  spatial offset

is 45 degrees and the  Faraday is 0 or 90 degrees (and very stable)

causing reciprocal conditions but 3 dB loss of  signal at both ends

of the QSO.   Sometimes  the  Faraday is  indeed  stable  enough to

permit complete QSOs in this case  (assuming both stations can deal

with the 3 dB loss).

The third exception is that it is possible for the  polarization

of a signal  passing  through the  ionosphere to become diffused or

elliptical.  In this case weak signals may be heard at a variety of

receive-antenna  polarizations,  but not  without some sacrifice in

signal strength.  Depending on the size of the two stations and the

exact nature of the  polarization  diffusion,  some QSOs are indeed

possible  in this  case  reguardless  of the  spatial  polarization

offset.  From observations made over a period of time it seems that

such  conditions  occur  more  frequently  at times  of high  solar

activity.

Again, the whole point is that it is less likely to complete EME

QSOs  when  the  spatial offset  is near 45 degrees  (and Max NR is

high) than when the offset is  closer to 0 or 90 degrees  (in which

case Max NR is low).    Therefore, a careful  consideration  of the

polarization issue when scheduling EME activities can significantly

improve the chances for success (by weeding out times that have the

lowest  chance  for  success),  but  will  never  offer an absolute

guarantee of anything.  While working on the Max NR concept for use

in Z-TRACK, many controlled tests were run with stations at various

spatial polarization offsets.  It was found that QSOs could be made

more frequently  and with less effort  when  the  offset  was  near

either end  of the scale  than when  it was  approaching 45 degrees.

Some QSOs were made in the  worst-case 45 degree  condition, but in

almost every case involved  significantly  weaker signals at one or

both ends of the QSO than when the offset was more favorable.

-------------------------------------------------------------------

Paul Kelley

PO Box 16

Dover-Foxcroft, ME 04426 USA

(207)  943-7484

E-mail: n1bug@mint.net

WWW: http://www.mint.net/~n1bug/

-------------------------------------------------------------------

1.  This equation was developed by C.H Hustig and was taken from an

article "Spatial  Polarization  and  Faraday  Rotation"  by Tim

Pettis, KL7WE.    This  article  appeared in PROCEEDINGS of the

22nd CONFERENCE of the CSVHF SOCIETY (Lincoln, NE  1988).

--------------------------------------------------------------------

Z-TRACK MaxNR Index              8                       Paul Kelley